# Broadcast

## 📢 What Is Broadcast in Networking?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

Broadcast is a one-to-all communication method where a device sends data to all other devices on the same network segment — whether they need it or not.

## 📦 Belongs to:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

✅ Types of Network Transmission Modes

Alongside Unicast (one-to-one) and Multicast (one-to-many)

## 🧠 Definition:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

In broadcast, the sender transmits a message to all devices in the local network (broadcast domain).

## 📡 Example Use Cases:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Scenario | Description |
| --- | --- |
| 🧭 ARP Requests | "Who has IP 192.168.1.1?" → all devices hear it |
| 📥 DHCP Discover | A new device says: "Any DHCP server out there?" |
| 📢 NetBIOS name resolution | Legacy systems ask the network: "Who is SERVER1?" |

## 🔢 IPv4 Broadcast Address

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Type | Address | Description |
| --- | --- | --- |
| Limited Broadcast | 255.255.255.255 | Sent to all devices on the local network |
| Directed Broadcast | e.g. 192.168.1.255 | Broadcast to all hosts in a specific subnet |

📌 Note: IPv6 does not use broadcast. It uses multicast instead.

## 🧰 Characteristics

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Property | Value |
| --- | --- |
| Direction | One-to-All (same subnet) |
| IP Level | Uses special broadcast addresses |
| MAC Level | Sent to MAC address FF:FF:FF:FF:FF:FF |
| Scope | Limited to local network (cannot cross routers) |

## ✅ Advantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

- ✅ Simple way to discover services (like DHCP)

- ✅ Useful for network booting, resolution, etc.

## ❌ Disadvantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

- ❌ Can lead to broadcast storms in large networks

- ❌ Wastes bandwidth — all devices process the message even if not needed

- ❌ Not routable — routers block broadcast traffic by default

## 📋 Comparison Table

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Mode | Type | Sent To | Use Case |
| --- | --- | --- | --- |
| Unicast | One-to-One | Specific device | Web browsing, file transfer |
| Multicast | One-to-Many | Subscribed group | Streaming, conferencing |
| Broadcast | One-to-All | All devices in local subnet | ARP, DHCP, NetBIOS |