# Multicast

## 📶 What Is Multicast in Networking?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

Multicast is a one-to-many communication method where a device sends data to a specific group of interested devices, rather than to all devices (broadcast) or just one (unicast).

## 📦 Belongs to:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

✅ Types of Network Transmission Modes

Alongside Unicast (one-to-one) and Broadcast (one-to-all)

## 🧠 Definition:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

In multicast, data is transmitted once by the sender and delivered to multiple selected recipients who have subscribed to a multicast group.

## 🧾 Examples of Multicast Use

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Scenario | Description |
| --- | --- |
| 📺 IPTV (Internet TV) | Stream sent once, received by many users who subscribe |
| 🧑‍🏫 Online webinars | Video/audio broadcast to multiple clients efficiently |
| 📡 Routing protocols like OSPF | Use multicast for neighbor discovery and updates |
| 📤 Software updates to groups | Efficient group data distribution (e.g., imaging labs) |

## 🧮 Multicast IP Address Ranges

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Protocol | Range | Notes |
| --- | --- | --- |
| IPv4 | 224.0.0.0 – 239.255.255.255 | Reserved for multicast groups |
| IPv6 | Starts with FF::/8 | Replaces broadcast in IPv6 |

Devices join multicast groups using protocols like IGMP (IPv4) or MLD (IPv6).

## 📌 How It Works

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

- Sender sends one copy of data to a multicast group address.

- Only devices that subscribed (joined the group) receive the data.

- Network switches/routers handle delivery efficiently, minimizing unnecessary traffic.

## ✅ Advantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

- ⚡ Efficient bandwidth use — sends one stream to many

- 📶 Scales well for many receivers

- 🎯 Targets only subscribers, not the whole network

## ❌ Disadvantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

- ⚙️ Requires multicast-aware routers/switches

- 🧪 More complex to configure than unicast

- ❌ Not widely supported on the public internet (used mostly in internal networks)

## 📋 Comparison Table

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Mode | Direction | Sent To | Use Case |
| --- | --- | --- | --- |
| Unicast | One-to-One | Single device | Web, email |  |
| Broadcast | One-to-All | All devices in subnet | ARP, DHCP |  |
| Multicast | One-to-Many | Subscribed group only | IPTV, video conference |  |

## 🔧 Protocols Using Multicast

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/9/2025]

| Protocol | Purpose | Multicast Address |
| --- | --- | --- |
| OSPF | Routing | 224.0.0.5, 224.0.0.6 |  |
| RIP v2 | Routing | 224.0.0.9 |  |
| IGMP | Group membership | Used by switches |  |
| PIM | Routing multicast | Builds distribution trees |  |